Enterprise Multi-Site Incident Response

TK002: ROAS Failure Between SiteB-R2 and ASW2 (VLAN 40 / SRV2)

| Field | Detail |
| --- | --- |
| Ticket | TK002 |
| Affected | SRV2 (192.168.4.2, VLAN 40, Site B), unreachable from all regional sites and the WAN core |
| Devices | ASW2 (trunk Fa0/4), SiteB-R2 (subinterface Gi0/0.40) |
| Root cause | VLAN 40 missing from ASW2’s trunk allowed-VLAN list |
| Status | Resolved |

# 1. Ticket description

“It has been reported from all regional sites and the WAN core HQ that end-hosts cannot reach SRV2 in regional Site B. However, the servers in the other regions are reachable, even by end-hosts in Site B.”

# 2. Initial symptom

C:\>ping srv2

Pinging 192.168.4.2 with 32 bytes of data:

Request timed out. (x4)

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

Every other host shows the same result. A timeout with no ICMP error message gives two hypotheses:

- The request is dropped on the way to SRV2, for example by an ACL, an ARP failure, a Layer 2 problem, or a server that is down.

- The request reaches SRV2 but the echo-reply is lost on the way back, for example because of an ACL, a host firewall, or a missing return route.

The scope narrows this down. Only SRV2 is affected, while the other servers answer from the same sources, so the common WAN and routing path is unlikely to be at fault. The fault is probably local to VLAN 40 in Site B. The layers are checked in order to prove it.

# 3. Troubleshooting (OSI Layers 1 to 4)

## Layer 1: Physical

ASW2# show interfaces status

Fa0/3   #VLAN 40#            connected  40     a-full  a-100

Fa0/4   #Uplink to SiteB-R    connected  trunk  a-full  a-100

SiteB-R2# show ip interface brief

GigabitEthernet0/0      unassigned    up  up

GigabitEthernet0/0.30   192.168.3.1   up  up

GigabitEthernet0/0.40   192.168.4.1   up  up

GigabitEthernet0/0.99   10.40.0.49    up  up

Finding: SRV2 sits in VLAN 40 on ASW2 Fa0/3, and its gateway is SiteB-R2 Gi0/0.40 (192.168.4.1). All the links are up, so Layer 1 is not the cause. Note that Gi0/0.40 shows up/up even though the fault turns out to be on this VLAN: a router subinterface stays up as long as the physical port is up, so up/up does not prove the VLAN has a working path.

## Layer 2: Data Link (fault found here)

ASW2# show vlan brief           -> VLAN 30, 40 (SRV2, Fa0/3), 99 and 999 are active

ASW2# show interfaces trunk

Fa0/4   on  802.1q  trunking  native 999

Fa0/4   Vlans allowed on trunk:                              30,99

Fa0/4   Vlans allowed and active in management domain:       30,99

Fa0/4   Vlans in spanning tree forwarding and not pruned:    30,99

ASW2# show interface fa0/4 switchport

Administrative Mode: trunk      Operational Mode: trunk

Trunking Native Mode VLAN: 999 (Native)

Trunking VLANs Enabled: 30,99

The VLAN exists and SRV2’s port is in it, but the only trunk carries VLANs 30 and 99. Compare the router’s subinterfaces with the trunk:

| SiteB-R2 subinterface | VLAN | In ASW2 trunk list? | Result |
| --- | --- | --- | --- |
| Gi0/0.30 | 30 (Production) | Yes | Works |
| Gi0/0.40 | 40 (SRV2) | No | Blocked |
| Gi0/0.99 | 99 (Management) | Yes | Works |

Because VLAN 40 is not allowed on Fa0/4, ASW2 never forwards VLAN 40 frames toward the router (and the router’s frames for VLAN 40 are dropped on arrival). SRV2 and its gateway are on the same VLAN but have no Layer 2 path between them. This also explains the scope: Site B users reach the other servers through VLAN 30, which is still allowed.

## Layer 3: Network

SiteB-R2# ping 192.168.4.2

.....   Success rate is 0 percent (0/5)

SiteB-R2# show arp

Internet  10.40.0.50   0  0002.4A5D.4116  ARPA  GigabitEthernet0/0.99

(no entry for 192.168.4.2)

SiteB-R2# show ip route 192.168.4.0

C  192.168.4.0/24 is directly connected, GigabitEthernet0/0.40

Redistributing via eigrp 2

- The ping fails from SiteB-R2 itself. Traffic originated by the router is not filtered by the Edge-R2 ACLs, and it never crosses the WAN. This rules out ACLs, redistribution, and the HQ routing path in one test.

- There is no ARP entry for SRV2. The router cannot resolve 192.168.4.2, because its ARP request on VLAN 40 never reaches the server. By contrast, VLAN 99 entries are present, which confirms the trunk works for the allowed VLANs.

- The routing table looks perfect, which is what makes this fault deceptive. The connected route stays installed and EIGRP AS 2 keeps advertising 192.168.4.0/24, so every other site keeps sending SRV2 traffic to SiteB-R2, where it is blackholed.

## Layer 4: Transport

A TCP test is not possible at this stage, because the failure occurs below Layer 3: with no ARP resolution there is no connectivity even for ICMP. The Layer 3 test from SiteB-R2 already excludes the Edge ACLs. After the fix, an HTTP request to SRV2 confirms that TCP traffic works as well as ICMP.

# 4. Root cause

VLAN 40 is missing from the allowed-VLAN list on ASW2’s trunk (Fa0/4). The router-on-a-stick subinterface Gi0/0.40 therefore has no Layer 2 path to SRV2, although both ends are individually healthy.

# 5. Resolution

ASW2(config)# interface fastethernet 0/4

ASW2(config-if)# switchport trunk allowed vlan add 40

The add keyword appends VLAN 40 to the existing list. Typing the list without it replaces the whole list, which is the most common way a VLAN gets removed from a trunk by accident.

# 6. Verification

ASW2# show interfaces trunk

Fa0/4   Vlans allowed on trunk:                              30,40,99

Fa0/4   Vlans in spanning tree forwarding and not pruned:    30,40,99

SiteB-R2# ping 192.168.4.2         !!!!!   Success rate is 100 percent (5/5)

C:\>ping srv2     (PC1, Site A)

Reply from 192.168.4.2: bytes=32 time=21ms TTL=122  (x4)

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

The TTL of 122 on the PC1 reply matches the expected path of six routers (SiteA-R1, Edge-R1, HQ-R1, HQ-R2, Edge-R2, SiteB-R2). Ping tests from the other sites and the WAN core also confirm the fix.

# 7. Lessons learned and recommendations

- An up/up subinterface proves nothing about the VLAN. Always compare each ROAS subinterface with the trunk’s allowed list (show interfaces trunk) when a single VLAN fails.

- Use switchport trunk allowed vlan add, never retype the full list, and check show interfaces trunk after every trunk change.

- Start with the scope. “One server unreachable from everywhere, others fine” points to a local, VLAN-specific problem before any ACL or routing check is needed.

- Test locally first. A ping from the nearest router separates local faults from WAN, ACL and routing faults in one step.