TK003 – Route Redistribution Failure

Enterprise Multi-Site Incident Response – Incident Report

| Ticket | TK003 – Site B users cannot reach any other site or the servers |
| --- | --- |
| Affected | Site B (VLAN 30 and VLAN 40), via SiteB-R2 and Edge-R2 |
| Symptom | Internal Site B traffic works; every destination outside Site B returns “Destination host unreachable” |
| Root cause | Edge-R2 has no mutual redistribution between OSPF 1 and EIGRP 2 |
| Fix | redistribute eigrp 2 subnets under OSPF and redistribute ospf 1 metric … under EIGRP |
| Layers checked | L1/L2 healthy → L3 faulty → L4 healthy |

1. Ticket and symptoms

Ticket TK003: “Site B users cannot reach any other site or the servers. Internal Site B traffic is fine. OSPF and EIGRP neighbors show as up.”

Tests from PC3 (Site B, VLAN 30):

| Destination | Result | Reading |
| --- | --- | --- |
| 192.168.6.2 (SRV3) | Timeout, then unreachable from 192.168.3.1 | Not reachable |
| 192.168.2.2 (SRV1) | Unreachable from 192.168.3.1 | Not reachable |
| 5.5.5.5 (Edge-R2 Loopback0) | 4/4 replies, TTL 254 | Reachable – inside Site B |

The “Destination host unreachable” replies come from 192.168.3.1, PC3’s own default gateway (SiteB-R2). A router sends this message when its routing table has no route to the destination. A failed ARP resolution would instead produce silent timeouts. So the suspicion falls on routing information, not on the local subnet. The working ping to Edge-R2 also shows that PC3 and the gateway are healthy and that the gap begins beyond Edge-R2.

2. Troubleshooting

Layers 1 and 2 – no fault

show ip interface brief on Edge-R2 and SiteB-R2 shows every used interface up/up, including the two ends of the 10.20.0.4/30 link (Edge-R2 G0/0 = 10.20.0.5, SiteB-R2 G0/1 = 10.20.0.6) and all three SiteB-R2 subinterfaces (G0/0.30, G0/0.40, G0/0.99). The two routers are directly connected with no Layer 2 device between them, so both layers are ruled out in one step.

Layer 3 – neighbors up, routes missing

Edge-R2 has an adjacency in each routing domain:

Edge-R2#show ip ospf neighbor

2.2.2.2     0   FULL/  -   00:00:33   10.20.0.1    GigabitEthernet0/1

Edge-R2#show ip eigrp neighbors

0   10.20.0.6    Gig0/0    13   00:00:23   40   1000   0   69

Both neighbors are healthy, which matches the ticket. Edge-R2 is also fully populated: it has OSPF routes learned from HQ-R2 (10.20.0.1) and EIGRP routes learned from SiteB-R2 (10.20.0.6). The routing tables on either side of Edge-R2 tell a different story:

| Router | What its table contains | What is missing |
| --- | --- | --- |
| SiteB-R2 | Only 5.5.5.5 (D), 6.6.6.6 and its connected subnets, and 10.20.0.0/30 (D EX) | Every OSPF-side route: loopbacks 1–4 and 7–13, all other sites, the servers |
| HQ-R2 | OSPF routes for HQ, Sites A, C and D | 192.168.3.0/24 and 192.168.4.0/24 (Site B) |

Each domain therefore knows nothing about the other. This breaks traffic in both directions: SiteB-R2 has no route towards the rest of the network (and no default route), and the rest of the network has no route back to Site B. The outbound failure shows first, which is why PC3 receives “unreachable” messages rather than plain timeouts.

Only one device sits on the boundary of the two domains, so the fault must be on Edge-R2. It is an ASBR (Autonomous System Boundary Router), not an ABR: it joins two different routing protocols, whereas an ABR connects several OSPF areas. Its running configuration confirms the diagnosis:

Edge-R2#show running-config | section router

router eigrp 2

eigrp router-id 5.5.5.5

passive-interface Loopback0

network 10.20.0.4 0.0.0.3

network 5.5.5.5 0.0.0.0

!

router ospf 1

router-id 5.5.5.5

log-adjacency-changes

passive-interface Loopback0

network 10.20.0.0 0.0.0.3 area 0

network 5.5.5.5 0.0.0.0 area 0

Neither routing process contains a redistribute statement. Compare this with the other Edge routers in this project, which redistribute ospf 1 into their site’s EIGRP process and that EIGRP process back into OSPF.

Root cause

Edge-R2 does not redistribute routes between OSPF 1 and EIGRP 2. Both adjacencies form normally because redistribution is independent of neighbor formation, so the neighbor tables looked healthy while no routes crossed the boundary.

3. Resolution

Redistribution is configured in both directions on Edge-R2:

Edge-R2(config)#router ospf 1

Edge-R2(config-router)#redistribute eigrp 2 subnets

Edge-R2(config-router)#router eigrp 2

Edge-R2(config-router)#redistribute ospf 1 metric 1000000 100 255 1 1500

- EIGRP into OSPF: subnets is required, otherwise OSPF redistributes only classful networks and drops the /24 and /29 subnets. No metric is needed because OSPF assigns a default seed metric of 20 (type E2).

- OSPF into EIGRP: EIGRP has no usable default seed metric, so one must be given. The five values are bandwidth in kbps (1000000), delay (100), reliability (255), load (1) and MTU (1500). Without them the routes are not redistributed.

4. Verification

Routes now cross the boundary in both directions:

| Check | Before | After |
| --- | --- | --- |
| HQ-R2 route to 192.168.3.0/24 and 192.168.4.0/24 | Absent | O E2 … [110/20] via 10.20.0.2 (Edge-R2) |
| SiteB-R2 routes to other sites | Absent (5.5.5.5 only) | D EX routes to all other sites, e.g. 192.168.6.0/24 via 10.20.0.5 |
| SiteB-R2 table size | Connected + 5.5.5.5 + 10.20.0.0/30 | 22 subnets with 4 masks, the same baseline as the other site routers |

The route type on HQ-R2 is O E2, not D EX: EIGRP routes that enter OSPF at Edge-R2 are re-marked as OSPF external type 2. The D EX type appears on the other side, on SiteB-R2, for OSPF routes that enter EIGRP.

HQ-R2#show ip route

O E2 192.168.3.0/24 [110/20] via 10.20.0.2, 00:06:06, GigabitEthernet0/1

O E2 192.168.4.0/24 [110/20] via 10.20.0.2, 00:06:06, GigabitEthernet0/1

SiteB-R2#show ip route eigrp

D EX 192.168.6.0/24 [170/28416] via 10.20.0.5, 00:07:20, GigabitEthernet0/1

End-to-end test from PC1 (Site A) to SRV2 (Site B) succeeds with 4/4 replies and TTL 122:

C:\>ping 192.168.4.2

Reply from 192.168.4.2: bytes=32 time=11ms TTL=122

Reply from 192.168.4.2: bytes=32 time=22ms TTL=122

Reply from 192.168.4.2: bytes=32 time=27ms TTL=122

Reply from 192.168.4.2: bytes=32 time=11ms TTL=122

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

Layer 4: SRV2’s web page opens from PC1, so no ACL blocks TCP traffic towards Site B. The ticket can be closed.

5. Lessons learned

- An adjacency in the “up” state proves only that two routers can talk. It says nothing about which routes they exchange.

- “Destination host unreachable” coming from the local gateway points to a missing route, while timeouts point to filtering, ARP or a missing return path.

- A boundary router needs redistribution in both directions. One direction missing gives one-way connectivity; both missing isolates the site.

- Compare the routing tables on both sides of the suspected boundary router, then read the running configuration of that router.