TK004 – DHCP Snooping Trust Failure

Enterprise Multi-Site Incident Response – Incident Report

| Ticket | TK004 – Site A (Sales) users cannot communicate with any other device |
| --- | --- |
| Affected | Site A, VLAN 10 (Sales) clients behind ASW1 |
| Symptom | PCs hold a 169.254.x.x (APIPA) address with no default gateway or DNS server |
| Root cause | DHCP snooping on ASW1 does not trust the uplink Fa0/4, so DHCP Offer and Ack messages from SiteA-R1 are dropped |
| Fix | ip dhcp snooping trust on ASW1 Fa0/4 |
| Layers checked | L1 healthy → L2 faulty (DHCP snooping) → L3 and L4 healthy |

1. Ticket and symptoms

Ticket TK004: “Site A (Sales) users complained that they cannot communicate with any other device on the network, even those in the same region.”

Tests from PC1 (Site A, VLAN 10):

| Test | Result | Reading |
| --- | --- | --- |
| ping 192.168.2.2 (SRV1, VLAN 20) | Request timed out | Different subnet, needs a default gateway |
| ping site-a-r1 | Host not found | No DNS server configured on the PC |
| ping 4.4.4.4 (SiteA-R1 Loopback0) | Request timed out | No path off the local subnet |

C:\>ipconfig

Autoconfiguration IPv4 Address..: 169.254.147.195

Subnet Mask.....................: 255.255.0.0

Default Gateway.................: 0.0.0.0

A 169.254.0.0/16 address is an APIPA address: the PC gave up waiting for a DHCP server and assigned itself one. It has no default gateway and no DNS server, so it cannot leave its own subnet. That explains every result above, including SRV1, which sits in VLAN 20 (a different subnet) even though it is in the same site. The DHCP server for VLAN 10 is SiteA-R1, so the question is where the DHCP exchange breaks between the PC and the router.

2. Troubleshooting

Layer 1 – links are up

On ASW1, Fa0/1 and Fa0/2 (VLAN 10), Fa0/3 (VLAN 20) and the trunk Fa0/4 towards SiteA-R1 are all connected. On SiteA-R1, G0/0 and its subinterfaces G0/0.10 (192.168.1.1), G0/0.20 and G0/0.99 are up/up. Layer 1 is not at fault.

Layer 3 – the DHCP server is configured but has no leases

SiteA-R1#show run | section dhcp

ip dhcp pool VLAN10

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

dns-server 192.168.6.2

SiteA-R1#show ip dhcp binding

IP address       Client-ID/              Lease expiration        Type

Hardware address

The pool is correct: right network, gateway and DNS server. Yet the binding table is empty, so no client ever completed the exchange. A binding is created only after the full Discover → Offer → Request → Ack sequence, so either the Discover never reaches the router or the Offer never gets back to the PC. The server side is healthy, so the next step is the path between them, the switch.

Layer 2 – VLANs, trunk, port security, DHCP snooping

ASW1#show interfaces trunk

Fa0/4   on   802.1q   trunking   999

Fa0/4   Vlans allowed on trunk: 10,20,99

ASW1#show port-security interface f0/1

Port Status : Secure-up      Violation Mode : Restrict

Total MAC Addresses : 1      Security Violation Count : 0

Last Source Address:Vlan : 0090.0CBC.93C3:10

- VLANs and trunk: Fa0/1 and Fa0/2 are in VLAN 10, and VLAN 10 is allowed and forwarding on the Fa0/4 trunk. The Discover can reach the router.

- Port security: Fa0/1 is secure-up with one learned MAC and zero violations. The MAC 0090.0CBC.93C3 matches the PC’s link-local address, so the port is not blocking PC1.

That leaves DHCP snooping, which is enabled on VLANs 10 and 20 in this project:

ASW1#show ip dhcp snooping

DHCP snooping is configured on following VLANs: 10,20

Interface                  Trusted    Rate limit (pps)

FastEthernet0/1            no         unlimited

FastEthernet0/4            no         unlimited

FastEthernet0/2            no         unlimited

This is the fault. Fa0/4, the uplink towards the DHCP server, is untrusted. DHCP snooping drops server messages (Offer, Ack) arriving on an untrusted port. The PC’s Discover goes out normally and SiteA-R1 answers, but the Offer comes back in on Fa0/4 and is discarded by the switch. The client never sees it and falls back to APIPA.

| DHCP message | Direction | Port on ASW1 |
| --- | --- | --- |
| Discover / Request | PC → router | Enters on Fa0/1 (untrusted): allowed |
| Offer / Ack | Router → PC | Enters on Fa0/4 (untrusted): dropped |

Rule: only ports that lead towards the legitimate DHCP server (here the uplink) are trusted. User ports stay untrusted so that a rogue DHCP server plugged into one of them cannot answer clients.

Layer 4 – no impact

DHCP uses UDP ports 67 and 68. The ACL on Edge-R1 does not permit them, but the DHCP exchange never crosses Edge-R1: client and server are both inside Site A, and no ACL is configured on SiteA-R1 or ASW1. The transport is not the problem.

3. Resolution

The uplink is trusted on ASW1:

ASW1(config)#interface f0/4

ASW1(config-if)#ip dhcp snooping trust

4. Verification

ASW1#show ip dhcp snooping

FastEthernet0/1            no         unlimited

FastEthernet0/4            yes        unlimited

FastEthernet0/2            no         unlimited

C:\>ipconfig /renew

IP Address......................: 192.168.1.11

Subnet Mask.....................: 255.255.255.0

Default Gateway.................: 192.168.1.1

DNS Server......................: 192.168.6.2

SiteA-R1#show ip dhcp binding

192.168.1.11     0090.0CBC.93C3           --                     Automatic

Fa0/4 is now trusted while Fa0/1 and Fa0/2 stay untrusted. PC1 receives an address, the correct gateway (192.168.1.1) and the DNS server (192.168.6.2) from the VLAN10 pool, and SiteA-R1 records the lease against PC1’s MAC address.

5. Lessons learned

- An APIPA address (169.254.x.x) means DHCP failed. Check the pool and binding table on the server, then everything between client and server.

- An empty binding table with a correct pool points to the exchange breaking in transit, not to the server.

- On a switch running DHCP snooping, always check which ports are trusted. Only uplinks towards the DHCP server should be.

- Check the DHCP-related ports on every access switch with snooping enabled (ASW1–ASW3 in this project).