TK005 – ACL Faults on Three Edge Routers

Enterprise Multi-Site Incident Response – Incident Report

| Issue | Device | Root cause | Fix |
| --- | --- | --- | --- |
| (a) | Edge-R3 | ACL Finance-Inbound applied outbound on G0/1 instead of inbound | Remove it from out, apply it with in |
| (b) | Edge-R2 | ACL Prod-Outbound permits TCP to port 22, but SSH replies come from port 22 | Replace the entry with permit tcp any eq 22 host 10.40.0.4 established |
| (c) | Edge-R1 | ACL Sales-Outbound entry 40 uses host 192.168.3.0, which matches a single address instead of the subnet | Rewrite entry 40 with 192.168.3.0 0.0.0.255 |

1. Ticket

Ticket TK005: “Three consecutive complaints were escalated to the Network Engineer: (a) Site C has no connectivity beyond its edge router, (b) the Network Engineer cannot SSH to Site B devices, (c) the Sales server pages (SRV1) do not load even though ping works.”

The three complaints are independent. In each case the interfaces are up and routing is intact, and the fault is found at Layer 4, in an ACL that is misplaced or mis-written. Each section follows the same order: symptom, Layers 1–3, Layer 4, fix.

2. Issue (a) – Site C has no connectivity beyond Edge-R3

Symptom

C:\>ping 7.7.7.7                      (Edge-R3 Loopback0)

Reply from 7.7.7.7: bytes=32 time<1ms TTL=254        4/4 replies

C:\>ping 192.168.6.2

Reply from 10.30.0.5: Destination host unreachable.   0/4 replies

A Site C PC reaches Edge-R3 but nothing beyond it. The “unreachable” message comes from 10.30.0.5, Edge-R3’s site-facing interface, so the packet arrived at Edge-R3 and was refused there.

Layers 1–3 – no fault

On Edge-R3, G0/0 (10.30.0.5, towards SiteC-R3) and G0/1 (10.30.0.2, towards the WAN) are up/up. The routing table holds a route to every subnet: OSPF routes learned from the WAN neighbor 10.30.0.1 (for example 192.168.6.0/24 via 10.30.0.1) and EIGRP routes from SiteC-R3 (10.30.0.6). Adjacencies and redistribution work. Unlike TK003, the router has a route to the destination and still refuses the packet, which points to an ACL.

Layer 4 – ACL applied in the wrong direction

Edge-R3#show ip interface g0/1 | include access list

Outgoing access list is Finance-Inbound

Inbound  access list is not set

Finance-Inbound is written to filter traffic arriving from the WAN, but it is applied outbound on the same interface. Site C’s own outgoing traffic is checked against entries meant for the opposite direction and falls to the final deny.

Resolution

Edge-R3(config)#interface GigabitEthernet0/1

Edge-R3(config-if)#no ip access-group Finance-Inbound out

Edge-R3(config-if)#ip access-group Finance-Inbound in

Edge-R3#show ip interface g0/1 | include access list

Outgoing access list is not set

Inbound  access list is Finance-Inbound

3. Issue (b) – SSH to Site B fails

Symptom

C:\>ssh -l yannick 6.6.6.6        (SiteB-R2 Loopback0, from the Network Engineer PC)

% Connection timed out; remote host not responding

The login prompt never appears. Only the Network Engineer (10.40.0.4) may SSH to the routers, so this PC is the only valid source.

Layers 1–3 – no fault

Interfaces are up, routing is intact, and pings to 10.20.0.6 (SiteB-R2) and 10.40.0.51 (Site B management VLAN) succeed. Layer 3 works, but TCP port 22 does not, so the filter is at Layer 4.

Layer 4 – the reply does not match the permit entry

Edge-R2#show access-lists

Extended IP access list Prod-Outbound

!(Omitted output)

550 permit tcp any host 10.40.0.4 eq 22

!(Omitted output)

deny ip any any

Prod-Outbound is applied inbound on Edge-R2’s site-facing interface (G0/0), so it filters everything that leaves Site B. The SSH request passes Prod-Inbound on the WAN-facing G0/1 and reaches SiteB-R2, but its reply is stopped by Prod-Outbound on the way back to 10.40.0.4. In an extended ACL the port written after an address belongs to that address, so entry 550 reads as follows, and the reply matches neither side:

| Entry | Matches | Real SSH reply |
| --- | --- | --- |
| permit tcp any host 10.40.0.4 eq 22 | Source any, destination 10.40.0.4 port 22 | Destination port is a random high port: no match, falls to deny ip any any |
| permit tcp any eq 22 host 10.40.0.4 established | Source port 22, destination 10.40.0.4, ACK or RST set | Match: permitted |

Resolution

Edge-R2(config)#ip access-list extended Prod-Outbound

Edge-R2(config-ext-nacl)#no deny ip any any

Edge-R2(config-ext-nacl)#no permit tcp any host 10.40.0.4 eq 22

Edge-R2(config-ext-nacl)#permit tcp any eq 22 host 10.40.0.4 established

Edge-R2(config-ext-nacl)#deny ip any any

The final deny ip any any is removed and re-added so that it stays last. Keeping the original sequence number (no 550, then 550 permit …) is a shorter alternative that leaves the deny untouched. established limits the entry to replies to connections that already exist.

C:\>ssh -l yannick 6.6.6.6

Password:

SiteB-R2>

The login succeeds and the Network Engineer reaches SiteB-R2.

4. Issue (c) – SRV1 web pages do not load

Symptom

C:\>ping srv1                         (from PC3, Site B)

Reply from 192.168.2.2: bytes=32 time=18ms TTL=122      4/4 replies

Ping works but the browser request to https://192.168.2.2 times out. Replies to ICMP are permitted by another entry, while TCP replies are not.

Layers 1–3 – no fault

Interfaces are up and the successful ping proves that routing works in both directions.

Layer 4 – the permit entry does not cover the subnet

Edge-R1#show access-lists

Extended IP access list Sales-Outbound

!(Omitted output)

40 permit ip host 192.168.2.2 host 192.168.3.0

Sales-Outbound is applied inbound on Edge-R1’s site-facing G0/0, so it filters traffic leaving Site A. It should let SRV1 (192.168.2.2) answer every host in Site B’s VLAN 30 (192.168.3.0/24). Entry 40 uses host 192.168.3.0, which matches only the single address 192.168.3.0 (the network address, used by no device). The web replies from SRV1 to PC3 do not match, and the ACL’s final deny drops them. The correct form is the network address with a wildcard mask.

Resolution and verification

Edge-R1(config)#ip access-list extended Sales-Outbound

Edge-R1(config-ext-nacl)#no 40

Edge-R1(config-ext-nacl)#40 permit ip host 192.168.2.2 192.168.3.0 0.0.0.255

Edge-R1#show access-lists

Extended IP access list Sales-Outbound

40 permit ip host 192.168.2.2 192.168.3.0 0.0.0.255 (3 match(es))

The entry now matches the replies, as its match counter shows, and SRV1’s page opens from PC3.

5. Lessons learned

- Placement: a name such as “Inbound” or “Outbound” does not set the direction. In this project the name describes traffic relative to the site: *-Outbound ACLs filter traffic leaving the site and *-Inbound ACLs filter traffic entering it, and both are applied with in on the Edge router. Check show ip interface <interface> | include access list first.

- Who sends the “unreachable”: a router that has a full routing table and still returns “unreachable” is refusing the packet with an ACL. A missing route gives the same message, as in TK003, so check the routing table to tell them apart.

- Replies reverse the ports: in an ACL that sees return traffic, the service port (22, 80, 443) is on the source side. A permit that matches the destination port only covers requests.

- `host` versus wildcard: host matches a single address. To cover a subnet, use the network address with a wildcard mask.

- Ping is not proof for TCP: an ACL can allow ICMP and still drop TCP, so test the actual service.